Project-Phase 2

Deadline: Day 09/08/2026 @ 23:59

[Total Mark is 15]

Computer Networks

IT351

Instructions:

- You must submit two separate copies (one Word file and one PDF file) using the Assignment Template on Blackboard via the allocated folder. These files must not be in compressed format.

- It is your responsibility to check and make sure that you have uploaded both the correct files.

- Zero mark will be given if you try to bypass the SafeAssign (e.g., misspell words, remove spaces between words, hide characters, use different character sets, convert text into image or languages other than English or any kind of manipulation).

- Email submission will not be accepted.

- You are advised to make your work clear and well-presented. This includes filling your information on the cover page.

- You must use this template, failing which will result in zero mark.

- You MUST show all your work, and text must not be converted into an image, unless specified otherwise by the question.

- Late submission will result in ZERO mark.

- The work should be your own, copying from students or other resources will result in ZERO mark.

- Use Times New Roman font for all your answers.

Student Details:                                            CRN:

| Name: SABA HASSAN ALANSARI Name: MANAR AHMED ALATA Name: SEREEN HASSAN ALELYANI Name: HOUR MOHAMMED ALJUWAYSIR |  | ID: 240023452 ID: 20030069 ID: 240007069 ID: 240028663 |
| --- | --- | --- |

15 Marks

Phase 2 Requirements

Testing and Verification

- Use the ping command to test connectivity between a device in any LAN of WAN 1 and a device in any LAN of WAN 2.

- Verify that data packets are successfully transmitted (0% packet loss).

Questions to Answer (10 Marks — 2 marks each)

Answer all five questions. For each question, you must include (a) a clear screenshot from your Packet Tracer project and (b) the commands used at that instance (e.g., IOS CLI commands or the configuration steps applied). Screenshots without the corresponding commands/steps will receive ZERO marks for that question.

- Creating nodes: Show a screenshot of the complete topology with all nodes created (12 end devices, 4 switches, 2 routers). List the device types selected and any steps/commands used to name the devices (e.g., hostname configuration). (2 marks)

Devices were added from the Packet Tracer device list:

- 12 PCs

- 4 Cisco 2960-24TT Switches

- 2 Cisco 1941 Router

- Giving connections in the LAN: Show a screenshot of the LAN connections between end devices, switches, and routers. State the cable types used for each connection and why. (2 marks)

Cable Type:

PC to Switch: Copper Straight-Through

Switch to Router: Copper Straight-Through

Reason:

Straight-through cables are used to connect different types of network devices

- Assigning IP addresses: Show screenshots of IP address, subnet mask, and default gateway assignment on at least one end device in each LAN, and the commands used to configure the router LAN interfaces (e.g., interface, ip address, no shutdown). (2 marks)

Enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface g0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

exit

- Creating the WAN connection: Show a screenshot of the serial link between Router 1 and Router 2, and the commands used to configure the serial interfaces and enable routing (static routes or a dynamic routing protocol such as RIP), including clock rate if applicable. (2 marks)

interface s0/1/0

ip address 10.0.0.2 255.255.255.252

clock rate 64000

no shutdown

Router:

interface s0/1/0

ip address 10.0.0.1 255.255.255.252

no shutdown

Static Routes:

ip route 192.168.3.0 255.255.255.0 10.0.0.1

ip route 192.168.4.0 255.255.255.0 10.0.0.1

Router:

ip route 192.168.1.0 255.255.255.0 10.0.0.2

ip route 192.168.2.0 255.255.255.0 10.0.0.2

- Successful data transmission using the ping command: Show screenshots of successful ping tests between a device in WAN 1 and a device in WAN 2 (both directions), with the exact ping commands used and the results (replies received, 0% loss). (2 marks)

Written Reflection — End of Project Report (5 Marks)

The project report must include a Reflection section (300–500 words) as the last section before the Project Artifacts section. This section must be written in the students' own words and should address some of the following:

- Describe one specific challenge or obstacle you encountered during this project and explain in detail how you identified and resolved it.

- Explain why you chose your specific approach/method (e.g., static routing vs. RIP, your IP addressing scheme) over at least one alternative you considered, and what tradeoffs informed that decision.

- If you used any AI tools during this project (e.g., ChatGPT, Claude, Copilot), disclose which tool(s), for what specific purpose (e.g., debugging, grammar checking, brainstorming), and how you verified or modified the output.

- What would you do differently if you were to redo this project with more time or resources?

Version History / Commit Log Requirements (Common to Both Reports)

Students must document and submit their project history/timestamps using a live, editable link:

- Written report: Submit a shared Google Doc or Word Online link (sharing set to "Anyone with the link can view") along with your final PDF report. The link must show the progress of the students' work with timestamps/screenshots.

- Code/data/configuration files: Submit Packet Tracer files (.pkt) and any configuration scripts via a GitHub or GitLab repository link (the coordinator may choose a repository suitable for the course). The repository must show a commit history reflecting incremental progress. Commit regularly (at minimum once per week), with clear, descriptive commit messages.

A separate section in the report must be added and named "Project Artifacts" to include all links, for example:

  - Live report document (with edit history): [link]

https://seuedu-my.sharepoint.com/:w:/g/personal/s240023452_seu_edu_sa/IQDP_KcRufixTrZz4DTTF39oATI07gldQ-sVsMWJvAV4_PE?rtime=wHCCm8X03kg

  - GitHub/GitLab repository (with commit history):

https://github.com/seba410/IT351

  - Packet Tracer project file (.pkt): [link or attached]

What NOT to do: Students must not write their report elsewhere and paste the final version in at the last m

inute. They also must not make a single commit containing the entire project. Both are treated as red flags regardless of the quality of the final work.

Important Notes

- AI use: Students may use AI tools (ChatGPT, Claude, Copilot, Gemini, etc.) only for brainstorming approaches, debugging small errors, or proofreading text. AI must not be used to generate analysis, write configuration logic, or produce substantial portions of the report or project files. Any permitted AI use must be disclosed in the Reflection section. Undisclosed AI use, or AI use beyond this scope (disclosed or not), will result in a zero grade for the entire project.

- Any submission lacking a valid, inspectable version/commit history, or showing evidence of AI generation or plagiarism, will receive a zero grade for the entire project.

- Failure to submit the mid progress report as required, or submission of reports found to be fabricated, copied, or AI-generated, will result in a zero grade for the entire project.

- The final report must be submitted as a PDF, together with the live document link, the repository link, and the Packet Tracer (.pkt) file.

- The edit history of the live document and the commit history of the repository must show incremental progress across both phases (Phase 1 and Phase 2).